# Theory of Operation

The card can run in either Z80 mode or Centurion mode. This is controlled by the dip switch at B6 and also the additional jumpers on this card J1, J2 and J3. J1 and J2 allow options for where functionality is bodged to hard wire for Centurion mode on the original. J3 is added to allow for more flexibility in ROM and RAM configurations.

An additional change has been made where the two 2114 1k x 4 SRAM ICs have been replaced by a 6116. Combined with the J2 jumper, 6116 ICs can be used on any of the 24 pin memory slots apart from address 0 which must be ROM for the card to boot.

In Z80 mode, backplane interrupts assert the INT* line and DMA requests assert the NMI* line on the Z80

Do Not set the card to Z80 mode while the CPU5/6 card is inserted. Bad Things will happen.

CPU5/6 mode is enabled by setting keys 5 and 8 open on the dip switch at UB6 and J1 tied to GND and J2 tied to +5V. Refer to the schematic for details. Z80 mode has these switches in the opposite settings.

## Memory Mapping (Z80 mode)

The first 28k of memory is available from the board memory sockets. With J3 tied to +5V these can be a mixture of 6116 RAM or 28C16 ROM chips. Technically 2716 ROMs could be used but my intention during the design was to use 28C16 ROMs which is what I’ve tested with.

From 28k to 32k the external (backplane) address space is used, which allows MMIO access. This is also the case from 60 to 64k. From 32k to 60k is either a mirror of the first 28k or can be set for external addresses.

The external addresses are controlled by the 74LS670 at UC5, accessible via IO addresses 30h to 33h setting address mapping for the respective 16kiB blocks in the Z80 address space. The low three bits of the data set the external address values for A15 to A17 and the fourth bit sets whether internal or external memory should be used for that block. Address mapping for external addresses is enabled by writing to IO address 21h and disabled by writing to IO address 20h.

Note that A14 is used to select high or low 16k within a 32k block and even numbered entries have A14 low and odd numbers A14 high. Note also that the high nibble is used for the current interrupt level so care must be used when setting an entry in the table.

An example mapping table is as follows:

| Block | Value | External 32k Block | Use External | Comments |
| --- | --- | --- | --- | --- |
| 0 | 0000b | 0 | No | Default, this should always be set to 0000b |
| 1 | 0111b | 7 | No | Used to access MMIO space or internal memory (A14 high) |
| 2 | 1000 | 0 | Yes | External 16kb memory from address 0h |
| 3 | 1000 | 0 | Yes | External 16kb memory from address 4000h (A14 high) |

## Backplane control signals (Z80 Mode)

Parity is toggled between even and odd by writes to IO Address 22h and 23h (note that even and odd parity are opaque to the card and I can never remember which way round the active low signal enables).

Backplane reset is asserted by a write to IO Address 25h and reset by a write to IO Address 24h.

Interrupt Acknowledgement is asserted by a write to IO Address 27h and reset by a write to IO Address 26h. See below for details of a bus interrupt.

## Bus Interrupt Cycle (Z80 Mode)

A bus interrupt is triggered by the Centurion IO device asserting the INTR* signal. This is tied to the Z80 INT* line. The Z80 needs to be set to IM 1 (interrupt mode 1). The interrupt handler needs to assert the interrupt acknowledgment by writing to IO address 27h (see above), read the requested level by reading the low nibble from IO Address 30h and set the current interrupt level register by writing to IO address 30h with the level in the high nibble and 0000b in the low nibble. Note that the IO address for setting the current interrupt level is shared with the memory map table so when writing to either of them the current values should be respected. By writing the interrupt level with Memory Map entry 0 in the table as default 0000b then the memory map won’t be affected. Writing other memory map entries when the interrupt level is not zero should be avoided.

Finally the interrupt acknowledge should be reset by a write to IO Address 26h.

## LED Displays and Dip Switch Settings (Z80 Mode)

Dip Switch settings are read from IO Address 10h. LED displays are written to IO Address 10h for the digit displays and 18h to 1fh for the decimal points (even values on, odd values off for the respective decimal point display). The displays are blanked by writing to IO address 17h and enabled by writing to IO address 16h.

## DMA Control (Z80 Mode)

DMA needs the start address and byte count available to the NMI handler. My intention is to use BC’ for the count and HL’ for the start address and swap the register sets in the handler. Contrary to the CPU5/6, the count can be stored non-inverted as it’s totally under the control of the Z80 assembly code.

Note that the address must be an external (i.e., Centurion backplane memory card) memory address.

The other CPU5/6 DMA instructions can be simulated by writing to the card registers.

| CPU5/6 DMA Instruction | Card Equivalent | Comment |
| --- | --- | --- |
| DMA SAD | LD HL,xxxxh or similar | Use alternate register set |
| DMA SCT | LD BC,xxxxh or similar | Use alternate register set |
| DMA SDV | OUT 10h to 13h | even addresses clear, odd addresses set bits 0 and 1 of the DMA device address |
| DMA EAB | OUT 15h |  |
| DMA DAB | OUT 14h |  |

The handler needs to set DMA active by writing to IO address 25h and reset at the end of block transfer by writing to IO address 24h.

The following is mostly an educated guess and needs to be tested.

Once DMA is active, repeatedly place the address on the bus. The memory cycle will be controlled by the IO device and the card logic.

One way of doing this is a loop with a CPI instruction which will increment HL and decrement BC. Exit the loop once BC is zero, then reset the DMA Active signal.

Note this doesn’t appear to be very failure tolerant and something bad happening will be in the middle of the NMI handler so there isn’t really any recovery option.

# Bill of Materials

Note: Horizontal mount 14 pin sockets for the test selection dip switch and TIL311 LED displays are available Preci-dip 299-83-314-10-001101. There may be other alternatives.

## Capacitors

| C1,C2 | 2 | 150µF 50V axial |
| --- | --- | --- |
| C3 | 1 | 47µF 50V axial |

35 x Bypass 100nF 2.54mm pitch ceramic disc as required

## Resistors

Refer to build notes for resistor value details

| R1,R2,R3,R4,R5,R6,R7,R8,R13,R14 | 10 | 4k7 |
| --- | --- | --- |
| R9,R10,R23 | 3 | 330R |
| R11,R12,R18 | 3 | 1k |
| R15,R16,R17 | 3 | Not populated, refer to build notes |
| R19,R20,R21,R22 | 4 | 470R |
| RP1,RP2 | 2 | 4k7 10 pin SIP |
| UK2 | 1 | 220/330 divider eg Bourns 4116R-3-221/331LF |

## Diodes

| CR1 | 1 | D [WIP check with David] |
| --- | --- | --- |
| CR2,CR3 | 2 | Not populated, refer to build notes |
| CR4 | 1 | LED |
| UL5,UL6 | 2 | TIL311 Single digit programmable LED |

## Switches/Jumpers

| J1,J2,J3 | 3 | 2.54mm pitch SPDT switch or 2.54mm pitch 1x3 jumper |
| --- | --- | --- |
| TP1,TP2,TP3,TP4 | 4 | TestPoint |
| P2 | 1 | 26 pin IDC horizontal |
| UB6 | 1 | DIPx08 |
| UL7 | 1 | DIPx07 |
| UK8 | 1 | Slide DPDT eg C&K F2UOA01AAU |

## Crystal

15MHz HC49-U

## Integrated Circuits

| UA1,UAB1,UC6 | 3 | 7438 See note |
| --- | --- | --- |
| UA2,UA3,UA4,UA5,UA6,UB3,UB4 | 7 | DS8835N |
| UA7,UB5,UK7 | 3 | 74LS373 |
| UA8 | 1 | AM2907 |
| UB1,UC4,UC8 | 3 | 74LS08 |
| UB2,UK1 | 2 | 74LS109 |
| UB7,UK3,UK4 | 3 | 7437 See note |
| UC1,UC7,UD8 | 3 | 74LS74 |
| UC2 | 1 | 74LS161 |
| UC3 | 1 | 74LS85 |
| UC5 | 1 | 74LS670 |
| UD1,UD7,UE8,UJ4 | 4 | 74LS04 |
| UD2 | 1 | 74LS688 |
| UD3,UD4 | 2 | 74LS244 |
| UD5 | 1 | Z84C00xxP |
| UD6,UJ6 | 2 | 74LS00 |
| UE1 | 1 | 74LS20 |
| UE2 | 1 | 74LS164 |
| UE3,UE4 | 2 | 74LS138 |
| UE6,UE7 | 2 | 74LS245 |
| UF1,UF2,UF3,UF4,UF5,UF6,UH1,UH2,UH3,UH4,UH5,UH6,UH7 | 13 | 28C16 |
| UF8 | 1 | 6116 |
| UH8 | 1 | 74LS155 |
| UJ1,UJ2 | 2 | 74LS259 |
| UJ3,UJ7,UJ8 | 3 | 74LS157 |
| UJ5 | 1 | 74LS10 |

Note on drivers: Centurion used 7437 and 7438 drivers. From what I can see 74LS37 and 74LS38 drivers have sufficient current sinking to drive the backplane bus which is terminated with a 220R pullup and 330R pulldown, as long as additional pullups aren’t populated. I used 7437 and 7438 drivers when I built up the board because that’s what I had in my parts bin.

## Mechanical

Ejector clip eg BIVAR CP-26 RD

# Build Notes

These build notes are how I went about putting the card together. It’s intended to guide, not instruct, there might be omissions and mistakes or you might want to take a different approach.

## Power and Clock

Solder C1, C2 and C3 to the board. Add test points TP2 and TP3 (optionally add TP1 and TP4 at this time).

Add UB7, R9 and R10 (located between B7 and B8). R9 and R10 can be any reasonable range, I used 330R but Centurion used a variety of values in their oscillators and if you want to standardise on 470R that would probably work too. Add the 15MHz crystal at location B8.

Power up the board either through the edge connector or through the test points and confirm 15MHz oscillation on pin 8 of UB7.

Add UD7, R11, UD8, UC8, UD8 and UE8. Confirm 5MHz signal on pin 8 of UC8 and inverted on pin 4 of UD7.

I used a jumper wire for R17, I haven’t worked out what should be populated for R15, R16, R17, CR2 and CR3 but I don’t think the circuit would be necessary if you are using a modern Z80. Confirm 2.5MHz signal on either pin of R17.

## Z80 Free Running

Add RP1, RP2 and UK2. Add UC4 and UD5 and a socket for UE6 and UB2 (optional at this stage, see below). Add R14 (I used 1k for R14, see IO Control section below).

If you have a 40 pin test clip, you can clip this over the Z80 and not worry about the sockets for UE6 and UB2 but if you’re socketing everything then you’d need to add them at some point anyway.

Use jumpers to temporarily tie pins 2 to 9 to GND on the UE6 socket. Use a jumper to temporarily tie pin 7 on UB2 to VCC (I used a 1k resistor in the UB2 socket).

You should now be able to power on and confirm the Z80 is free running.

Note: From this point on, you need to be very careful about powering the card from the CPU5/6 backplane. If you are going to do this, then all other cards including the CPU card(s) should be removed unless you have a good understanding of what you are doing.

## Memory selection

Add UB6 and R12. Set key 8 to GND (closed position) on the dip switch.

Add UD3, UD4, CR4 LED and R23. I used a 330R resistor for R23, but 470R would probably work as well, but a bit dimmer. Or 220R or 180R for brighter.

Add J1, UE3 and UE4. Jump J1 middle pin to GND. Note the bypass capacitor pads are quite close to the jumper pads.

Power up and check select pins on UE3 and UE4.

## Memory control read

This starts to get a bit chaotic. I combined the read and write steps and went straight to the repeated memory test when I built up. The following description is how I was planning to build and test but I got impatient.

Add UJ1, UJ2, UJ3, UJ4, UJ5, UJ6, UJ7 and UJ8. Add UD6 and UH8. Add socket only for UC5. Add J2 and J3.

Set J3 middle pin jumped to UJ4 pin 10. Add temporary jumper from UC5 pin 6 to GND.

Add J2, set middle pin jumped to UC4 pin 3. I had some issues with noise on J2 output, check there is no short to adjacent traces.

[WIP maybe more sockets and jumpers]

Add UE7 and remove the jumpers from UE6 and populate the ‘LS245.

Change setting on J1 to jump middle pin to pin 12 of UJ7/pin 19 of UE7

Populate UF1 socket. Jumper D0 to D7 to GND and test free running and verify chip select, output enable and write enable on UF1.

Program a 28C16 EEPROM with NOPs followed by a JP 0000 to return to the base start of ROM. Verify free running with ROM installed in UF1.

## Memory Control Write (WIP)

Add UF8 socket. Note that this is different from the original where there were two 2114s for the RAM. I made some changes so that in theory you can interchange the 28C16 chips with 6116 to mix the combination of RAM and ROM, but I haven’t tested this.

Make a ROM to write to UF8 and verify the CS, OE and WE signals on UF8.

Populate UF8 with a 6116. Make a ROM to repeatedly write and read back from the 6116 and HALT if there’s a difference. HALT anyway after a certain number of tests (after some seconds to verify the code is running correctly and not just randomly halting) to test the HALT LED. Reset the card by grounding the common pin at K8 to repeat the test.

[WIP need to check the timing of the WE signal with data on the bus and see if there’s an issue with data before WE] [Seems to work but not aligned with the datasheets]

## IO Control (WIP)

Very chaotic at this point

Add UK7, UL5, UL6, UL7, UA7, UA8, UC5, UD1, R3, R4, R6, R7, R8, R19, R20, R21, R22.

Note the 4k7 value for the Centurion bus pullups has been selected to ensure the drivers on IO cards aren’t overloaded. Centurion used 7437/8 drivers or equivalent which can sink 48mA or more depending on the driver. The bus lines seem to be terminated by a 220R pullup or higher in most cases so any reasonable value above 220R should be fine, after confirming the IO card drivers. The pullups on the bus lines (where not on the original) are only required if you’re planning on using the card in Z80 mode. I used 220R when I built the board.

Run whatever tests you need to do at this stage. Read the DIP switch and use its value to put some output on the LED displays. Write to the output registers and check their outputs.

## Bus Interface (WIP)

Add UA1, UA2, UA3, UA4, UA5, UA6, UC6, UB1, UB2, UAB1, UC1, UC2, R1, R2, R18.

Run whatever tests you need to do at this stage. Set the page mapping up for MMIO. Read the Boot ROM at addresses FC00h to FDFFh for the 512 byte ROM or FD00h to FDFFh for the 256 byte ROM. Display the contents slowly on the LEDs.

Interface with the MMIO card. Wait for character input. Provide some character output. Write an interrupt handler and set the MMIO card up for interrupts. Test it works.

Do a memory test on the 32k or 128k memory card.

## DMA Interface (WIP)

Add UK1, UK3, UK4 and P2.

Run whatever tests you need to do at this stage. Read track 0 on the floppy drive.

## CPU5/6 mode (WIP)

Add the remaining chips and resistors.

Run whatever tests you need to do at this stage. Burn some original ROMs and run the diag tests.

Notes:

IO Addresses

IO Read (IN A,(N))

10 Read Dip Switches

30 Read Control Register

Bit 0-3: INR0-3 (Active Low)

Bit 4: DRDY (Active High)

Bit 5: APRE (Active Low)

Bit 6: DMA Interrupt (Active Low)

Bit 7: Parity Error (Active Low)

IO Write (OUT A,(N))

00 Clear DMA Device bit 0

01 Set DMA Device bit 0

02 Clear DMA Device bit 1

03 Set DMA Device bit 1

04 Disable DMA

05 Enable DMA

06 Enable LED output

07 Disable LED output

08 Clear Decimal 1

09 Set Decimal 1

0a Clear Decimal 2

0b Set Decimal 2

0c Clear Decimal 3

0d Set Decimal 3

0e Clear Decimal 4

0f Set Decimal 4

10 Write to LEDs

20 Disable Paging

21 Enable Paging

22 Even Parity (maybe other way round)

23 Odd Parity (maybe other way round)

24 Disable Backplane Reset (active low)

25 Enable Backplane Reset (active low)

26 Disable IACK (active low)

27 Enable IACK (active low)

28 Disable DMA Active (active low)

29 Enable DMA Active (active low)

2a Disable DMA End (active low)

2b Enable DMA End (active low)

30 Write Page/Int Level

31 Write Page/Int Level

32 Write Page/Int Level

33 Write Page/Int Level

Paging/Memory

There is 28kiB of memory available on the board. This memory is mirrored between 0-28kiB and 32-60kiB. This allows ROMs to be created at origin 8000h and be used by the Z80 on startup without extra hardware.

Paging can be used to allocate a 32kiB block of CPU5/6 memory into either the upper or lower half of the address space (various configurations are possible depending on the ROM/RAM population on the board). Note that the Z80 interrupts require that the zero page has interrupt handlers for both the maskable and non-maskable interrupt.

The top 4kiB of memory is always considered external. This can be used to access the MMIO addresses. A page table entry is needed to .

The page table entry has an active flag to indicate that the CPU5/6 memory should be used instead of the on board ROM/RAM. This is only relevant if Paging is enabled on the board through software.

DMA

Haven’t quite figured this out yet…DMA is always through CPU5/6 memory. Z80 is the DMA controller with bit banging. The DMA interrupt basically needs to set the data and do a read or write. The DMA control code must be on the card, i.e., not in CPU5/6 memory.

Logic Analyser:

Pod 4:8-0 Pod 3:8-2 Address

Pod 3:1-0 Pod 2:8-3

Pod 2:2 MREQ

Pod 1:7 IORQ

Pod 1:6 INT

Pod 1:5 NMI

Pod 2:1 RD

Pod 2:0 WR